ECE 3235 Electronics II

	Experiment # 4


Updated on Oct 1st, 2014.
Introduction to Cadence Tool
Note: (1) Cadence Tool is arguably the most comprehensive set of electronics design assistance and design automation tools.  It is widely used in industry. Also, it will be used in more advanced courses such as ECE4311 VLSI Design and ECE5211 Advanced Analog Integrated Circuit Design. (2) It looks like a very long lab, but the instructions are only a few pages long if excluding the figures. (3) The software will be used throughout the semester, so please spend time to get familiar with it.
1. General information

In this lab, we will use Cadence software to build the schematic for the circuit shown below, and then simulate the circuit.  You might have used some other similar tools, like SPICE, HSPICE, PSPICE (OrCAD).  Cadence software has the most comprehensive set of tools for circuit design and is widely used in industry and academia. Though you may only use some of the Cadence tools in this course, it is a good starting point.
The circuit shown below is a RLC circuit, which you have studied in ECE2006.  
[image: image1.jpg]- Virtuoso® Schematic Editing: mylib_rlc rlc schematic - Affirma Analog Circuit Design Environment (1)
cmd: E Status: Ready T=27 ¢ Simulator:

Design Window Check Options

chiilousePopUp () hiRepe




2. Start using Cadence
Part I Schematic Editor
2.1 Setting up the Cadence Environment

One important thing is that Cadence software mostly runs in the UNIX/LINUX environment (operating systems). Since the computers in MWAH 293 runs only windows 7, therefore before you can run Cadence, you need to emulate the UNIX system. Do the following: 
1. Make sure the computer is connected to the internet. (The student might have to login in online since the computers are on the wireless network.)
2. Click the start.
3. Click programs. Find the software named Open Text Exceed 14×64.

4. Click on the software and the menu expands and find Exeed Tools. Click on it and find Exceed XDMCP Query. Click on it. 
5. A window pops up. In the host field, enter cdsm.d.umn.edu and the click ok.

6. Then a login window to the UNIX systems appears. Login to the server with your username and password (username is the login name of your university email, and password is ‘welcome’).
After you login, right click your mouse and select Open Terminal. 
(1) In the appeared window, from the terminal prompt (typically cdsm.d.umn.edu%), create a new directory from your home directory called cadence.  This is where you’ll organize all your Cadence files and directories. (Note: if you are NOT familiar with UNIX OS, please do the following commands with TA’s guidance. There is a space between words. ~ below stands for the home directory where you are when you just log in).
cdsm.d.umn.edu% tcsh

> mkdir
~/cadence 
(2) Copy the dotfiles .* to your home directory (do not omit the . before *).
> cp
/zpool1/research/thua/ece3235_test/.*
~/ 
(3) Type the following command:

> source
~/.cshrc 
(4) Copy the file cds.lib to your working directory (overwrite if asked) 
> cp
/zpool1/research/thua/ece3235_test/cds.lib
~/cadence/ 
(5) Create a sub-directory to the cadence directory, called Mylibs.
> mkdir
~/cadence/Mylibs 

(6) Create a sub-directory to the cadence directory, called Models.

> mkdir
~/cadence/Models 
(7) Then copy a model file to your directory Models.

$ cp
/zpool1/research/thua/ece3235_test/Models/allModels.scs
~/cadence/Models

Now, close your Terminal window from the upper left menu and Open a new Terminal window. Then, proceed to Section 2.2 below.
2.2 Start Cadence 
From a command prompt, type: 
cdsm..d.umn.edu% tcsh
> source
.cshrc (always do this before the next command)
To start Cadence, type: 
> cd cadence
> virtuoso
&  (do not omit the & sign)
The program appears after a few moments. Do not worry about the “error” shown in the window, as long as you see “END OF USER CUSTOMIZATION”.  You can then close the “What’s new” window unless you really want to know what’s new.  You window would look like this:
[image: image2.jpg]Virtuoso® 6.1.3 — Log: /zpool 1 /research/thua/CDS.log

Eile Tools Options Help cadence

&
Loading ans. cxt

Virtuoso Framework License (111) was checked out suscessfully. Total checkout tine was 0.01s
*Errort eval: undefined function - EXKickEN

“Errort strcat: arguent #1 should he either a string or a syubol (type template = "S) - nil
END OF USER CUSTOMIZATION

T
nmouse





Add the library path
On the window menu, select Tools(Library Manager, then a new window appears.
Under File, click New, then click Library…, type mylib_rlc in the name filed of the appeared window.  Then select the directory to be Mylibs created before.  The window should look as shown below.
[image: image3.jpg]Library

Name mylib_ric

Diectory | portssearcherecsszas test [ ¢l e (6l
El
Woels
=
3 aivodels scs
3 casin
3 voManager.iog
3 ibManager.log.casick

Fie oot Pt

Design Manager

< Use NONE
_ Use No DM





Then click ok.  A new window appears and select the option “Do not need process information” as follows:
[image: image4.jpg][~ Technology File for New Library

Technalogy File for ibrary "mylib_ric"

Voucan: « Compile an ASCH technology fle

 Reference exsting technology libraries

 Atiach to an existing technology library
Do not need process information

[ ox DTS





Then click ok.  The steps above created a new library called mylib_rlc (check the existence in the library column of the Library Manager window).
2.3 Creating a new Design

From the Library Manager, click mylib_rlc once to highlight, select File( New(Cell View, and fill in the form as shown below in order to define the new rlc schematic cell view:

- In the Cell Name field, type rlc 

- In the Type selection, choose Schematic. (It should automatically define the View Name as schematic). Click OK. 
[image: image5.jpg]File
Library yio_ric @

Cell rle
View schenatic
Type schematic B

~Application

Open with schematics L @

 Always uss this application for this type of fle

Library path fle
“2pooll/research/thua/ece3235_test/cds. 1ib

[ox (TN





Note that a small window asking about whether you want to use a different license may appear, simply click yes.  Then, an empty Schematic Editor Window with dots on the background now appears, where you will be drawing your circuit schematic as you have similarly done before in PSPICE. 
2.4 Schematic Editor
The process of editing a design is called schematic capture. You can use several methods in the Cadence environment tools to achieve the same effect. We could select from the pull-down menus, or click on one of the icons, or use a shortcut letter, referred to as a Bindkey. 

Browse through the icons at the top of the Composer Window, and note the “floating description” on each. 

2.5 Placing the Instances 

Click on the Instance Icon or select Create(Instances… (you may also notice that the shortcut letter for this is “i”).  An add instance window appear as follows. 

[image: image6.jpg]Library Browse

cell

View symbol

Names.

aray Rows Calurns

(@2Rotste ) (dhsiveways ) (S upside Down)

[ e e





Click the Browse button in the form. The Library Brower – Add Instance window appears as shown below. 

[image: image7.jpg]__ Show Categories

Library cell View
analoglin res symbol
(=T Tveees ucd
ahdiLin pvcesp auLvs

pucvs hspiceD
basic pyevsz spectre
casDefTechLib pvevsd
functional pvevsp
mylin_ric rcwireloat
passiveLin
MExamples scasubckt
Lib scoes

scovs

schoky

ser

simulinkCoupler

spltswitch

spatswitch

spatswitch

spatswitch

svees i

I_L Close ) (Eiters.. ) (Display... ) (Help )






Select the following in that window as shown above:
- Under the Library column, select analogLib. 

- Under the Cell column, select res (the short name for resistor)
- Under the View column, select symbol
After the selection above, the previous Add Instance window will be displayed as shown below. Before you click on the schematic window to place the resistor symbol, edit the Add Instance form by modifying the Resistance value to 22k Ohms, as shown below in the window: (as you can see, there is a lot of other parameters associated with the resistor, but leave all the rest empty for now)
[image: image8.jpg]Library "analogLib Browse

cell res

View  symbol

Narmes

Aray Rows 1 Colunns 1

(4ZRotste ) (AhSideways ) (& Upside Down)

Model name

Resistance 22K Ohms

Lengih

Widn
Muliplier

Scale factor

Temp ise from ambient

Temperature coeficient 1

Temperature coeffcient 2

Resistance Fom

Generate noise? B
Capasitance

alias fr Lin. emp. co

Alias for Quad temp. co.

Lintemp co of lin cap

Qua temp €0 of lin cap

Resistance Scaling Factor
Capacitance Scaling Factor

AC resistance

Temperature difierence

QD (Cncel) (st ) (risip




- Now click in the schematic editor window (with dots in the background) to place the resistor. (If the above Add Instance window blocks your view, you can select Hide in that window before you place the resistor).
- Another resistor symbol follows the cursor. Place it in the window then click on Cancel on the Add Instance window. The form disappears.
- Add the other instance symbols from the analogLib as indicated below, 

Add an instance of capacitor from analogLib, (named cap) and set it to 47nF,
Add an instance of inductor from analogLib (named ind) and set it to 500mH

Add an instance of Ground from analogLib (named gnd). 
Place the above symbols in the order as shown in the following window

[image: image9.png]\Virtuoso® Schematic Editor L Editing: mylib_rlc rlc schi| -

ow cadence

mouss L schsingle i schiDel ChiiMousEPopUR0
ED





Note that the instances you placed in the schematic editor window may not be in the orientation as shown, but you can rotate them.  Click on the menu Edit(Rotate and then click on the instance you want to be rotated.
2.6 Adding the Input/Output Pins
We need to add pins to define the input and output. To add the input and output pins, Click on the menu Create(Pin. The Add Pin form appears. 
- Under Pin Names, type Vin. Note that Direction in the form reads input, as shown below.
[image: image10.jpg]Pin Names vin

Direction input B3 s Expansion @ ot L on
Usage schematic | Flacement e single _ mutiple

Aftach Net Expression: @ No o Ves

Property Name

Default Net Nane

Font Height 0.0625 Fontstyle  [stick [

| 2Rotste ) (AhSideways ) (& Upside Down) (_Show Sensttivty > |
@Y (_cancel )(_Defauts J(_Help )|






- Go back to the Schematic Editor window and click once in the window.  Note that the pin named “Vin” is placed in the position you specified.
- Now, invoke the Add Pin window again.  Type Vout this time, change Direction to read Output (to define the output pin). Place the Vout pin in the Schematic Editor window. Press ESC to clear the form.
After the above steps, your Schematic Window would like this:

[image: image11.jpg]Virtuoso® Schematic Editor L Editing: mylib_ric ric schematic A=
ptions  Migr cadence

. vout

mouse L mouseAddPI) chHiMousePapUpQ

\
[16)] s e optons form o suppy nd:Pin




2.7 Connecting Wires

The instances in the Schematic Editor window have not been connected yet.  To connect the wires, click on the icon Wire(narrow) or click Create(wire. 
- then click on one end and the other end to make wire connection
- Wire the components as shown below:
[image: image12.jpg]Virtuoso® Schematic Editor L Editingﬂylibirlc rlc schematic

iptions  Migr o cadence

AB0PI) e Draw Made chHiMausePapUp)
ing point for th





2.8 Modifying Instance Properties 

To modify the properties of each instance, click once to select each individual instance, then click on the “q” key (query) on your keyboard. This is the same as selecting the instance, right-clicking using the mouse and then selecting the Properties form. 


- If some of your instances in the Schematic window do not have the same values as shown in the above window, you can then change their values by doing the above step. By default, Cadence automatically writes the Instance Name. 

2.9 Checking and Saving 

To check and save the schematic, click on the design icon on the left, Check and Save or click on File(Check and Save.  Notice the Cadence’s Command Interpreter Window (CIW) (the window that shows a letter C when minimized) displays the following messages:

Extracting “rlc schematic"

Schematic check completed with no errors. 

”mylib_rlc rlc schematic” saved.

If you get Warnings/Errors: 

- Go back to your schematic and look for the “flashing” tiny squares. 

- Browse back in the CIW history window and read the warning/error messages. 

- Fix the warnings/errors as necessary. Warnings are not as crucial as Errors. 

- For example “Dangling Wires” Warnings could be ignored. 

- Click on the Check & Save icon, and repeat until the design has no errors. 

2.10 Creating the Symbol Cellview 

Now we’ll create a symbol (black box) to represent our circuit. The symbol view will be created based on the already-available schematic view. This is called creating a Cell view from another Cell view. (For example, go to Library Manager window, you see three columns as Library, Cell and View.  We already have a view called schematic for the cell rlc, now we will create another view called symbol for the same cell rlc.  In general, there are multiple views for the same cell, which act like different representations).
- From the Schematic Editor window, select Create(Create Cellview (From Cellview...

This sequence creates symbols automatically, based on their primary input and output pins. The Cellview From Cellview form appears as shown below. 

[image: image13.jpg]Cellview From Cellview

Library Name nylib_rle Browse

Cell Name e

From View Name fschematc [ To ViewName  [syabol

Tool /DataType [schematicsymbol g

Display Cellview &
Edit Options v

T Cancel) Detauts ) apply ) _Help)




- Change “Tool/Data Type” to SchematicSymbol if this is not the default, then click OK.


- On the new form that appears, select the “Load/Save” button. The form expands. 

- In the form’s lower left section, change the cyclic field next to “Load” to “Analog”. 

- Click on the “Load” button to load the Analog Symbol Generation Template as follows, then click OK.
[image: image14.jpg]Library Name Coll Name View Name

Ebtin 5 S

Pin Specitcatons ttes
Lenpins an @
RghtPits vout um}
Top Pins

Botiom Pins List
Exclude Inherited Cannection Pins:

© None &l L Only these,

Load/save ¥ Edit Atiibutes Edit Labels _ Edit Properties

Loaw/Save Symbol Template Configuration

Load ) (znaiog @ 74c613/t0ls. s0186/dETT /samples /symbolten/artist. tog

Save ) TSG File

Load Template Data From Symbol

Library Name Cell Name View Name
synbol

[ ox D TR




- If a message appears calling to “Overwrite Base Cell CDF”, click on No so as not to overwrite the inherited parameters in the base cell Component Description Format. A new Symbol Editor window appears 

- In the Symbol Editor Window, the outside red box defines the selection region when selecting the symbol if it’s present on a schematic design. The inside green box defines the dimension of the symbol as it would show when placed in a schematic design. Expand both, and drag and drop the square pins, the cdsName, and the connecting terminals as shown in figure below:
[image: image15.jpg]cadence

RLE

cdsTerm(”im ) s Brfartd me()
cdsParam(2 erm(*vout')
cdsParam(3 "






The cdsTerm(“Vout”) is a label that displays the pin names or the net names (Vout). The cdsParam(1,2,3) are labels that display parameters of an instance, e.g. 75 Ohm. The cdsName(“”) is a label that displays the instance or cell name, e.g. rlc.

- From the Symbol Editor window, Click on Create(Label…

- An Add Symbol Label window appears, type RLC in the Label field. 

- Press Enter to place the label. Click on Cancel on the form to close it. The symbol

should look as that in the window above with “RLC” symbol.
- Now press the Save icon in your design window. The CIW should display the message: "rlc symbol" saved. 

- From the pull-down menus of the Symbol Editor window, select Window ( Close. 

- The Schematic Editor window should be still open. Close it using Window( Close.

The produced symbol will be used in the cell test_rlc to be created next. This is a circuit model that’s used to test the performance of a design. It will incorporate input signal sources, power, ground, the circuit load (e.g a capacitance), and the positive and negative supply rails (buses). 

2.11 Creating the test circuit (test_rlc) 

We’ll now create a new cell using the rlc symbol created before as one of its instances. 
- From the Library Manager Window (open it if you do not have it open), highlight mylib_rlc in the Library column, go to: File( New( Cell view... 

- Create a new Cell called test_rlc, select Schematic in the type field if it is not
- Click OK. A new Schematic Editor window appears. 

- Connect your new circuit using components according the schematic figure shown below.
- To add RLC to the editor, select Create(Instance…. In the appeared Add instance window, browse library to mylib_rlc and select rlc in cell column and then symbol in View column.  Then place it in the Schematic Editor window.  Click Cancel to close the Add instance window.
-  To add the voltage source, again select Create(Instance…. In the appeared Add instance window, browse library to analogLib, select vsin in the Cell column and then symbol in the View column. Notice that a property window appears for the voltage source (vsin here stands for a sinusoidal voltage source).  
!!IMPORTANT!!
Input 0 for the DC voltage field, 1 for AC magnitude field, 50m in Amplitude field, 1M in frequency field and leave all others unchanged.  Please note that the units of measure like V, Hz will follow automatically in the fields.
- Repeat the above steps to add a capacitor and a ground as shown in the window below.

- After all instanced are placed, connect them with wires to complete the circuit.  Please go back to previous pages for how to connect wires if needed. 
- After you make the wire connections, click on the wire connected to the vsin so that it is highlighted, then right click, select Add name.  An add wire name window appears, type Vin in the names field, then go to Schematic Editor window and place the name on that wire.  In this way, that wire will have a name, which is easy to control later.  Similarly, give the name Vout for the output wire connected to the load capacitor. 
-. Finally, the test schematic should look like below:

[image: image16.jpg]cadence

R sehHiMousePopUR0)
| Crud: wire 1]





- Click on the Check and Save icon or File(check and save when done.
Part II Simulation in ADE (Analog Design Environment)
2.12 Initializing the Simulation Environment 

In part I, we have built the circuit schematic. The next step is to simulate the circuit, so that designers can verify the functionality of the circuit (which is typically the design process). Before starting the simulation, make sure that your test_rlc schematic is open. 
- In the test_rlc Schematic Editor window, select Launch(ADE L. In a few moments, the Vituoso Analog Design Environment window appears, as shown below. 
[image: image17.jpg]Session Setup Analyses Variables Outputs Simulation Results Tools Help

I Status: Ready T=27 C _ Simulator: spectre

Design Variables Analyses

Name | TyRe. [Enabie |

Outputs
Name/Signal/Expr

(I

B
| T ot ter Simulation: (2o @ Pioting mode: Bepizce [

4| setup Outputs





Again, here it may ask for the license, just click on yes. Also, another window titled What’s New in Analog/Mixed Signal appears.  Close it to avoid too many windows unless you really want to read it.

The icons on the right provide quick access to frequent commands/menus. The title of the window lists Library, Cell, and Cell View of the design being simulated (it is mylib_rlc test_rlc schematic in this case). The Analyses Area: Lists the types of analyses, any arguments (i.e. time interval), and weather it’s enabled to perform the simulation in the current run (it is empty now because we have not specified any run yet). The Design Variables Area: Lists components set as variables, i.e., a Capacitor, C1, varying from 1uF to 1pF (empty now as well). The Outputs Area: Lists names of nets/signals/expressions/ports to be plotted on the output waveform window (also empty now). 
The following sections will provide more detailed descriptions on how to run simulation. 
2.13a. Choosing a Simulation Engine

- We’ll select the simulation engine to be Spectre. In the window, select Setup(Simulator/Directory/Host

- In the appeared window as follows, ensure the Simulator cyclic field is reading Spectre 

[image: image18.jpg]Choosing Simulator/Directory/Host — Virtuoso® Analog |

Sinulator spectre B

Project Ditectory  ~/esdence/simmlation @

Host Mot @ local  remote o distibuted

Hast

Retiste Diecty @

T Cancel) _Detauis ) _agpiy ) _ielp





- Change the Project Directory to read ~/cadence/simulation. This creates a new directory under your cadence folder.

-. Click ok.  If you’re prompted to “save the current state”. Click Yes, and confirm the name as state1 in the form that opens afterwards. Click OK. If not, just proceed.
2.13b. Plugging in models

- In the Analog Design Environment window, select Setup (Model library. In the appeared window as follows, click on Browse button on the right hand side, go to the directory Models and select the file allModels.scs, click on open and then notice that the model file is added in the window.
[image: image19.jpg]spectrel : Model Library Setup

Er Global Model Files





Click on ok to dismiss the window.

2.14 Choosing the Analyses 

- In the Analog  Design Environment window, click the Choose Analysis icon. Instead, you could select it from Analysis ( Choose pull down menu. The Choosing Analysis form appears as follows:

[image: image20.jpg]| ~|enoosing Analyses —— Virtuoso® Analog Desig|

fralysis  etan  Lde oac  noise
wn © sens o dematch o st
o o e s
Upa upsth o pnose o px
Cpsp wpss wgpac o opnoise
Cogpd wopsp o hb © hbac
 hhnoise

Transient Analysis

Stop Time,

Accuracy Defaults (errpreset)

_ conservative __ moderate _ liberal

| o J

Enatied _ optons.. )
DD (Cncel) st ) (ppiy) _ein )





We’ll simultaneously set up several analyses modes in this window.
• Transient analysis: This provides the transient output response of the circuit with respect to time. The user specifies the time period and the time variant input waveform while the simulator calculates the output response.

• AC analysis: This simulates the AC performance of the circuit as a function of frequency, and is based upon the small-signal frequency response model.

• DC Operating Point: This analysis simply determines the D.C. operating point of the circuit based on the parameters present on the schematic assuming all capacitors opened and all inductors shorted. It is the default mode and is automatically performed before any other analysis in order to determine the initial state of the circuit.

• DC sweep mode: This generates DC transfer characteristics for the circuit by varying a user specified independent source over a range of values. 

2.15 Transient Analysis 

1. In the Analysis Section, select tran.

2. Set the Stop Time field to 3u. 

3. Turn on the Enabled field (at the lower left corner of the window). 

4. Click APPLY. (do not click OK yet as we need to set up more analyses) 

Notice that in the Analog Design Environment window, under the Analysis Section, a line was listed to describe this analysis. 

2.16 AC Analysis

1. In the Analysis Section, select ac. The form expands as follows:
[image: image21.jpg][~ |Choosing Analyses — Virtuoso® Analog Desig|

Analysis  Ltan e e ac  noise
vt o sens o dematch o st
o us e opss
Cpa upsh o pnose o pd

wpsE wgpss Logpac o gpnoise
woapd wgpsp b  hbac
 hbnoise.

AC Analysis

Sweep Variatle
® Frequency

 Design Variable
 Temperature
 Component Parameter
 Made! Parameter

Sweep Range

@ Start-Stop
 Center-Span

Stat 0.0k Stop 10k

Sweep Type
 Puints PerDecade 5o

T 20
Logarithnic  Number of Steps

Add Specific Points

Specialized Analyses
Nane B

Enahled v





2. Set the Sweep Variable to Frequency(note the other sweep variables). 

3. Set the Sweep Range to Start-Stop. (Start: 0.01k, Stop: 10k). The units of measure is automatically Hz so you do not have to put in Hz. 

4. Set the Sweep Type to Logarithmic, with 20 Points Per Decade. 

5. Turn on the Enabled field.

6. Click on Apply.

Notice in the Analog Design Environment window, under the Analysis Section, a line was added to describe this analysis.

2.17 DC Sweep and DC Operating Point

1. In the Analysis Section, select dc.  Note that the form changes accordingly.
2. In the Sweep Variable section, select Component Parameter.  Note the form changes accordingly.
3. Click on Select Component. This allows for selecting the instance on the schematic. 

4. Click on the supply source from the Schematic window. 

5. A form appears listing all the instances parameters. Select the dc parameter in the first line. Then click OK to close the form.
6. In the Sweep Range section, select Start-Stop. (Start: 0, Stop:100) 

7. Turn on the Enabled field. 

8. Click OK this time. When done, the form should appear as shown next.

[image: image22.jpg]|—|Choosing Analyses —— Virtuoso® Analog Desig|

Analysis  Utan  ede Lo o noise
Y o sens w dematch o sth
o oS wenvp s
e oopsth o proise o pdl
PSP oOpss U ogpac o gpnoise
o oapsp o hb o hbac.
o ftnoise

DC Analysis

Save DC Operating Point
Hysterssis Sweep i

Sweep Variable

Component Nare /v

_ Temperaturs
__ Design Variable

 Component Paramster  Parameter Nare

_ Model Parameter

Sweep Range

@ Start-Stop
 Center-Span

stat [0

Sweep Type

[putomate |3

Add Specific Points

Enabled v

Stop

D (cace) st

Select Component

4

100

|__Optians.
el

Apply ) (_Help





Notice in the Analog Design Environment window, under the Analysis Section, a line was added to describe this analysis.  The final look of the Analog Design Environment window should be as shown below.

[image: image23.jpg]Session Setup Analyses Variables Outpus Simulafion Results Tools Help

I Status: Ready T=27 C _ Simulatar: spectre

Design Variables Analyses
Name | [ Type [Enabe | Arguments

i oc ¥ 0100 Automatic Stat-Stop V0

2 »  1010K 20 Logarithnic Points Per Decade

v 0w

e

Outputs
Name/Signal/Expr__ [Value |FIot |Save | Save Options. |

Plot afer Simuiation: (2uts__ @ lotting mode: Feplace |





2.18 Saving and Plotting Simulation Data 

The simulation environment is configured to save all node voltages in the design by default. You can modify the default to save all terminal currents also, or you can select specific set of nodes to save. We’ll select these nodes from the schematic window.

- Select Output ( To be Plotted ( Select on Schematic. Note the circuit Schematic window will jump in front of you for your selection. Node voltages can be selected by clicking on the wire in the schematic window, and currents by clicking on the terminals. Unselecting can be performed either by clicking on the terminal/node again, or by selecting the corresponding line in the Outputs section of the Analog Design Environment window and clicking on the Delete icon. In this lab, select the input and output wires of the test_rlc circuit. Observe the simulation window as the wires get added as follows:
[image: image24.jpg]Session Setup Analyses Variables

I Status: Selecting oupuls to be plotted.

Outputs Simulation Results Tools Help

T=27.C_ Simulatar: spectre.

Design Variables Analyses
TR R - 7 — |eslisieial Enable:] s— I

lioc v 0100 Automatic Stan-stop /v

e v 1010k 20 Logaritmic Points Per Decate

R CE]

Ouputs

[ Name/SignallExpr _|Value | Flot |Save | Save Options |

Jy vin v ally

[g vout v ally

& i B

i 2

Plot afer sinuiation: (2uts__ @) lotting mode: eplace [

> Select on Schematic Ouputs to Be Plotted

4| choose Design





2.19 Running the Simulation - The Waveform Window 

- Click on the Netlist and Run icon on the right (the Green button).  Alternatively, select Simulation ( Netlist and Run.
- When it complete, the plots are shown automatically in a window. The 3 Analysis Responses are shown in separate sub-windows as follows. 
[image: image25.jpg]| Active : mylib_ric test_ric schematic : Feb 16 00:36:02 201013 | - | ||

10

File Edit Frame Graph Ads Trace Marker Zoom Tools Measurernents Help cadence
SRHBOFEGE S DX e Z]abel
a AC Responss g 5 Response P
[— [— [v—

1000

s

75
750
& g
g 5 2
5 & g
500
P
250
75 25 0o
o 5 10 15 20 25 a0 ol e o 00 250 %0 750 10
a7l area ™ [zl Tov Szl szearsey





Notice that another window also appears, which shows you the status of the simulation. You can simply close that window. The DC response is not clear due to the voltage divider effect.

We’ll next modify the appearance of the displayed waveforms.

- Click anywhere in the DC Response area to work in this section. Notice the highlighted box reading “3” at the top-right corner of the section.

- Select Axes ( Strips, or click on the Switch Axis Mode icon on the left. Observe the 2 separate plots displayed. Repeat for the 2 other plots (1, 2).

The windows then changes as follows:
[image: image26.jpg][~ Ac C tes 6 00:36:0 B

File Edt Frame Graph Axs Trace Marker Zoom Tools Measwrements Help cadence

[=2a% FEE S 2 BX¢sE

Transient Responze ] AC Response 3] DC Response B

[]abel

1000

v

50

75
0 & 101520 28 30
5 r

o
10!

oo
00 250 500 750 100

[Trasssus w5 0w Seosml  tov ™ ssoul oV





- Now we use the markers to measure the output peak-to-peak amplitude of the Transient Response signal. Click on the Marker(Place(Trace Marker, then place the marker on the positive peak of the output waveform.  Then select Marker(Add Delta, and place the marker at the negative peak.  Notice the data appears in the window, which shows the time difference and voltage difference between two points we selected.

- Select the AC Response section. Add a Trace Marker as we did above. Pull the marker up, till it can’t go up any further. Read the resonance frequency from the data at the bottom of the window. 
!!IMPORTANT!!

Now, please spend some time to look into the simulation results. Do the results make sense to you?
Note that for Transient Response, the two plots in that column shows the circuit input (labeled as /Vin in the plot) and output (labeled as /Vout) changes as a function of frequency.  Verify the input sinusoidal input waveform, is this what you specify for the voltage source vsin in the test_rlc circuit schematic? Measure the frequency of the input sin waveform, is the frequency what you specify for the voltage source vsin? The output waveform resembles a sinusoidal waveform as well, but the amplitude is so small compared to the input, why?
Now go to the AC response column. Again, two plots are shown there.  The upper plot shows what the output is as a function of frequency (please clearly notice the x-axis is frequency as this is AC analysis). The lower plot shows what the input is as a function of frequency. Notice it is horizontal line. Why this is so? Is the magnitude 1.0V what you specified for the voltage source vsin in the teset_rlc circuit schematic? Why don’t we specify smaller magnitude such as 1mV, because AC analysis is small-signal analysis and the signal magnitude is suppose to be “small”? You can change it to 1mV and run the simulation again.  What is the difference? So, does it really matter between 1mV and 1V?
Finally, please go to the DC response column.  The upper plots show what the output DC voltage is as a function of input DC voltage. Notice the x-axis is the DC voltage we swept, as we did set up to sweep the DC voltage in DC analysis previously.  For example, when input DC voltage is 100V, the output DC voltage is shown as about 340mV, is this correct?  The lower plot shows what the input DC voltage is as a function of input DC voltage.  So, of course, that would be a straight line.  If you want to delete the bottom Vin-Vin plot (as we are typically interested in the Vout-Vin plot) in the DC Response Column, click on the bottom waveform, and press the Del key on your keyboard. 

2.20 saving the analyses setting and simulation results

Now, before you close the Analog Design Environment window, you may want to save the analyses setting and simulation results first, as next time you will not need to do it again.  Click Session(Save State and a window appears as follows:

[image: image27.jpg]Saving State —

Virtuoso® Analog Di

esign Environment (1)

Save State Option

 Dirsctory o Cellview

Directory Options

State Save Directory o/

artist_staced]

Save As statel

Existing States

Cellview Options

Library mylin_rc. B
cell test_rle ﬁy Bose
State spectre_statel g

Description

What to Save

¢ Analyses
£ Model Setup

¢ Sinulator Options

¢ Graphical Stimull

¢ Device Checking Setuy
 Distibuted Processing

¢ Variables
¢ Sinulation Files
 Convergence Setup
¢ Conditions Setup

p ¥ Cosimulation Options
 Parameterization Setup

_Select All ) (_Clear All_
+ Outputs

 Environment Options
£ Waveforn Setup

¢ Results Display Setup
 Turbo and Parasitic Reduction

D cace ) oy ) o)




A default state name called state1 is given, but you can change it to a different name if you want.  Then, you can click ok.  Next time, if you want to simulate the circuit again, you only need to click Session(Load State to re-load the analyses setting and simulation results.  As you probably have noticed, we can save multiple states for the same circuit.
Part III What to submit up to now?
1) A print of the circuit schematic of rlc?
2) A print of the circuit schematic of test_rlc?
3) The output waveform from simulation?
Part IV Building up an OpAmp model
In this part, you have the chance to build up an Operational Amplifier model using resistor, capacitor, diodes, VCVS (voltage controlled voltage source) etc. The model your are going to build is called a behavior model or a macromodel for the real OpAmp. The models include many of the non-idealities of real OpAmp, but does not include all of non-idealities. You will be using this close-to-ideal OpAmp in the rest of the course to simulation OpAmp based various amplifier circuits.

1) Build up the OpAmp model as shown in the next page (attached online). Select all the components from analogLib. Make a print of it.
2) Build up the inverting amplifier as shown in the next page (attached online). First, you have to create a symbol for the OpAmp model you make in 1) and then plug in here in the test circuit. Make a print of it.
3) Do an AC frequency analysis for the inverting amplifier and make a print of the output waveform.
PAGE  
1

