https://learn.adafruit.com/drive-a-16x2-lcd-directly-with-a-raspberry-pi

# MCP230xx GPIO Expander on the Raspberry Pi

Add 8 or 16 extra GPIO pins to your pia via I2C

http://crumpspot.blogspot.be/2013/05/using-3x4-matrix-keypad-with-raspberry.html

#include <Password.h>
#include <LiquidCrystal.h>
#include <Keypad.h>
LiquidCrystal lcd(2,3,4,9,10,11,12);
Password password = Password( "4321" );
const byte ROWS = 4; // Four rows
const byte COLS = 3; // Three columns
// Define the Keymap
char keys[ROWS][COLS] = {
  {'1','2','3',},
  {'4','5','6',},
  {'7','8','9',},
  {'*','0',' ',}
};
// Connect keypad ROW0, ROW1, ROW2 and ROW3 to these Arduino pins.
byte rowPins[ROWS] = {25, 24, 23, 22}; //connect to the row pinouts of the keypad
byte colPins[COLS] = {28, 27, 26}; //connect to the column pinouts of the keypad
const int buttonPin = 7;
int buttonState = 0;

// Create the Keypad
Keypad keypad = Keypad( makeKeymap(keys), rowPins, colPins, ROWS, COLS );

#define ledPin 13

void setup(){
   pinMode(buttonPin, INPUT);
lcd.begin(16, 2);
digitalWrite(ledPin, LOW); // sets the LED on
Serial.begin(9600);
keypad.addEventListener(keypadEvent); //add an event listener for this keypad
keypad.setDebounceTime(250);
}

void loop(){
keypad.getKey();
buttonState = digitalRead(buttonPin);
if (buttonState == HIGH) {
lcd.clear();
}
}

//take care of some special events
void keypadEvent(KeypadEvent eKey){
switch (keypad.getState()){
case PRESSED:
lcd.print(eKey);
switch (eKey){
case ' ': guessPassword(); break;
default:
password.append(eKey);
}
}}

void guessPassword(){
if (password.evaluate()){
digitalWrite(ledPin,HIGH); //activates garaged door relay
delay(500);
digitalWrite(ledPin,LOW); //turns off door relay after .5 sec
lcd.print("VALID PASSWORD "); //
password.reset(); //resets password after correct entry
delay(600);
lcd.print("Welcome");
delay(2000);
lcd.clear();
}


else{
digitalWrite(ledPin,LOW);
lcd.print("INVALID PASSWORD ");
password.reset(); //resets password after INCORRECT entry
delay(600);
lcd.clear();
}
}

http://www.instructables.com/id/Password-Lock-with-Arduino/step4/Code/

http://www.electronics123.net/amazon/projects/Electronics123_Door_Lock_Project/Arduino_Door_Lock.html

http://www.newtechandme.com/arduino/arduino-tutorial-lesson-3-2-keypad-and-lcd-with-password/

http://www.newtechandme.com/arduino/arduino-tutorial-lesson-3-1-keypad-and-lcd/

https://sites.google.com/site/arduinomega2560projects/home/level-3/keymembrane-and-lcd-password-lock

https://github.com/rohankapoorcom/ElectronicLock       door lock

// Python code for detecting Rising Edge

import RPi.GPIO as GPIO

import time

import os

#adjust for where your switch is connected

buttonPin = 17

GPIO.setmode(GPIO.BCM)

GPIO.setup(buttonPin,GPIO.IN)

while True:

#assuming the script to call is long enough we can ignore bouncing

if (GPIO.input(buttonPin)):

#this is the script that will be called (as root)

os.system("python /home/pi/start.py")

- import RPi.GPIO as GPIO           # import RPi.GPIO module

- GPIO.setmode(GPIO.BCM)            # choose BCM or BOARD

- GPIO.setup(port_or_pin, GPIO.OUT) # set a port/pin as an output

- GPIO.output(port_or_pin, 1)       # set port/pin value to 1/GPIO.HIGH/True

- GPIO.output(port_or_pin, 0)       # set port/pin value to 0/GPIO.LOW/False

- import RPi.GPIO as GPIO            # import RPi.GPIO module

- from time import sleep             # lets us have a delay

- GPIO.setmode(GPIO.BCM)             # choose BCM or BOARD

- GPIO.setup(24, GPIO.OUT)           # set GPIO24 as an output

- try:

- while True:

- GPIO.output(24, 1)         # set GPIO24 to 1/GPIO.HIGH/True

- sleep(0.5)                 # wait half a second

- GPIO.output(24, 0)         # set GPIO24 to 0/GPIO.LOW/False

- sleep(0.5)                 # wait half a second

- except KeyboardInterrupt:          # trap a CTRL+C keyboard interrupt

- GPIO.cleanup()                 # resets all GPIO ports used by this program

- import RPi.GPIO as GPIO            # import RPi.GPIO module

- from time import sleep             # lets us have a delay

- GPIO.setmode(GPIO.BCM)             # choose BCM or BOARD

- GPIO.setup(24, GPIO.OUT)           # set GPIO24 as an output

- try:

- while True:

- GPIO.output(24, 1)         # set GPIO24 to 1/GPIO.HIGH/True

- sleep(0.5)                 # wait half a second

- if GPIO.input(24):

- print "LED just about to switch off"

- GPIO.output(24, 0)         # set GPIO24 to 0/GPIO.LOW/False

- sleep(0.5)                 # wait half a second

- if not GPIO.input(24):

- print "LED just about to switch on"

- except KeyboardInterrupt:          # trap a CTRL+C keyboard interrupt

- GPIO.cleanup()                 # resets all GPIO ports used by this program

http://raspi.tv/2013/how-to-use-interrupts-with-python-on-the-raspberry-pi-and-rpi-gpio-part-3

interruption

import time

import _thread

import RPi.GPIO as GPIO

GPIO.setmode(GPIO.BOARD)

GPIO.setup(11, GPIO.OUT)

def wait_for_input(stop):

input("Press enter to stop blinking.")

stop.append(True)

print("Stopping the blinking.")

def blink():

stop = []

_thread.start_new_thread(wait_for_input, (stop,))

while True:

GPIO.output(11, True)

time.sleep(2)

GPIO.output(11, False)

time.sleep(2)

if stop: break

blink()

GPIO.cleanup()

http://crumpspot.blogspot.fr/p/keypad-matrix-python-package.html

http://maniacallabs.com/category/dial-a-song/

https://www.cl.cam.ac.uk/projects/raspberrypi/tutorials/robot/face_detection/

http://usualpanic.com/category/raspberry-pi/

http://tronixstuff.com/2013/12/16/arduino-tutorials-chapter-42-numeric-keypads/

http://musicdiver.com/wordpress/2013/01/keypad-code-lock-with-arduino/

Pour tous ceux qui aiment raccorder des capteurs, des boutons, des écrans et autres accessoires, c’est une bonne nouvelle : il y a  9  broches GPIO supplémentaires

processeur 700MHz Broadcom BCM2835 CPU

http://www.raspberrypi-spy.co.uk/wp-content/uploads/2014/07/Raspberry-Pi-GPIO-Layout-Worksheet.pdf

- Pin 7 on the keypad is ROW 0- GPIO 18

- Pin 6 on the keypad is ROW 1- GPIO 23

- Pin 5 on the keypad is ROW 2- GPIO 24

- Pin 4 on the keypad is ROW 3- GPIO 25

- Pin 3 on the keypad is COL 0 - GPIO 4

- Pin 2 on the keypad is COL 1 - GPIO 17 22

- Pin 1 on the keypad is COL 2 - GPIO 22 10

Super intéresant

http://raspi.tv/2013/how-to-use-interrupts-with-python-on-the-raspberry-pi-and-rpi-gpio-part-2

import RPi.GPIO as GPIO

GPIO.setmode(GPIO.BCM)

# GPIO 23 & 24 set up as inputs. One pulled up, the other down.

# 23 will go to GND when button pressed and 24 will go to 3V3 (3.3V)

# this enables us to demonstrate both rising and falling edge detection

GPIO.setup(23, GPIO.IN, pull_up_down=GPIO.PUD_UP)

GPIO.setup(24, GPIO.IN, pull_up_down=GPIO.PUD_DOWN)

# now we'll define the threaded callback function

# this will run in another thread when our event is detected

def my_callback(channel):

print "Rising edge detected on port 24 - even though, in the main thread,"

print "we are still waiting for a falling edge - how cool?\n"

print "Make sure you have a button connected so that when pressed"

print "it will connect GPIO port 23 (pin 16) to GND (pin 6)\n"

print "You will also need a second button connected so that when pressed"

print "it will connect GPIO port 24 (pin 18) to 3V3 (pin 1)"

raw_input("Press Enter when ready\n>")

# The GPIO.add_event_detect() line below set things up so that

# when a rising edge is detected on port 24, regardless of whatever

# else is happening in the program, the function "my_callback" will be run

# It will happen even while the program is waiting for

# a falling edge on the other button.

GPIO.add_event_detect(24, GPIO.RISING, callback=my_callback)

try:

print "Waiting for falling edge on port 23"

GPIO.wait_for_edge(23, GPIO.FALLING)

print "Falling edge detected. Here endeth the second lesson."

except KeyboardInterrupt:

GPIO.cleanup()       # clean up GPIO on CTRL+C exit

GPIO.cleanup()           # clean up GPIO on normal exit

GPIO.add_event_detect

GPIO.wait_for_edge

import RPi.GPIO as GPIO
GPIO.setmode(GPIO.BCM)
GPIO.setup(23, GPIO.IN, pull_up_down = GPIO.PUD_DOWN)
GPIO.setup(24, GPIO.IN, pull_up_down = GPIO.PUD_UP)
while True:
if(GPIO.input(23) ==1):
print(“Button 1 pressed”)
if(GPIO.input(24) == 0):
print(“Button 2 pressed”)
GPIO.cleanup()

import RPi.GPIO as GPIO
GPIO.setmode(GPIO.BCM)
GPIO.setup(23, GPIO.IN, pull_up_down = GPIO.PUD_DOWN)
GPIO.setup(24, GPIO.IN, pull_up_down = GPIO.PUD_UP)
def printFunction(channel):
print(“Button 1 pressed!”)
print(“Note how the bouncetime affects the button press”)
GPIO.add_event_detect(23, GPIO.RISING, callback=printFunction, bouncetime=300)
while True:
GPIO.wait_for_edge(24, GPIO.FALLING)
print(“Button 2 Pressed”)
GPIO.wait_for_edge(24, GPIO.RISING)
print(“Button 2 Released”)
GPIO.cleanup()

GPIO.remove_event_detect(23)

http://hardware-libre.fr/2013/07/ajouter-un-bouton-dextinction-avec-python/

pour le lcd

http://www.rpiblog.com/2012/11/interfacing-16x2-lcd-with-raspberry-pi.html

faire clignoter une led 50 fois pendant 1 seconde

- import RPi.GPIO as GPIO

- import time

- # blinking function

- def blink(pin):

- GPIO.output(pin,GPIO.HIGH)

- time.sleep(1)

- GPIO.output(pin,GPIO.LOW)

- time.sleep(1)

- return

- # to use Raspberry Pi board pin numbers

- GPIO.setmode(GPIO.BOARD)

- # set up GPIO output channel

- GPIO.setup(11, GPIO.OUT)

- # blink GPIO17 50 times

- for i in range(0,50):

- blink(11)

- GPIO.cleanup()

https://learn.adafruit.com/downloads/pdf/drive-a-16x2-lcd-directly-with-a-raspberry-pi.pdf

http://www.raspberrypi-spy.co.uk/2012/07/16x2-lcd-module-control-using-python/

Enable

R/W

RS

D4/D5/D6/D7

Clavier 4*3 = 4 lignes * 3 colonnes soit 7 gpios

Total : 14

GPIO.setup(LCD_E, GPIO.OUT)  # E

GPIO.setup(LCD_RS, GPIO.OUT) # RS

GPIO.setup(LCD_D4, GPIO.OUT) # DB4

GPIO.setup(LCD_D5, GPIO.OUT) # DB5

GPIO.setup(LCD_D6, GPIO.OUT) # DB6

GPIO.setup(LCD_D7, GPIO.OUT) # DB7

Super :    LCD avec I2C  //// http://www.recantha.co.uk/blog/?p=4849

http://michlstechblog.info/blog/raspberry-pi-connecting-a-hd44780-display-with-i2c-bus/

http://tech2077.blogspot.fr/2012/06/running-hd44780-lcd-over-i2c-on.html

SUPER à voir    http://edsard.nl/projects/raspberry/  LCD avec I2C

Projet intéressant :::::::     http://ikanbilis.com/16.php

http://www.raspberrypi-spy.co.uk/2013/07/how-to-use-a-mcp23017-i2c-port-expander-with-the-raspberry-pi-part-3/