- Alarme hors service : désactive (disabled) la sortie donc GPIO est à 3.3V

- Mise en service : (enable) active la sortie et fixe un niveau Low

http://plomteuxkevin.wordpress.com/2013/03/16/les-gpio/

http://jeremyblythe.blogspot.co.uk/2012/06/motion-google-drive-uploader-and.html

http://www.classthink.com/2014/01/12/raspberry-pi-vocal-intruder-warning-system-project/

#!/usr/bin/env python

import time

import RPi.GPIO as GPIO

def main():

# tell the GPIO module that we want to use the

# chip's pin numbering scheme

GPIO.setmode(GPIO.BCM)

# setup pin 25 as an output

GPIO.setup(23,GPIO.IN)

GPIO.setup(24,GPIO.OUT)

GPIO.setup(25,GPIO.OUT)

GPIO.output(25,True)

while True:

if GPIO.input(23):

# the button is being pressed, so turn on the green LED

# and turn off the red LED

GPIO.output(24,True)

GPIO.output(25,False)

print "button true"

else:

# the button isn't being pressed, so turn off the green LED

# and turn on the red LED

GPIO.output(24,False)

GPIO.output(25,True)

print "button false"

time.sleep(0.1)

print "button pushed"

GPIO.cleanup()

if __name__=="__main__":

main()

le programme ci-dessous utilise le mode interruption pour capturer l’évènement « appui sur bouton »

#!/usr/bin/env python

import time

import RPi.GPIO as GPIO

# handle the button event

def buttonEventHandler (pin):

print "handling button event"

# turn the green LED on

GPIO.output(25,True)

time.sleep(1)

# turn the green LED off

GPIO.output(25,False)

# main function

def main():

# tell the GPIO module that we want to use

# the chip's pin numbering scheme

GPIO.setmode(GPIO.BCM)

# setup pin 23 as an input

# and set up pins 24 and 25 as outputs

GPIO.setup(23,GPIO.IN)

GPIO.setup(24,GPIO.OUT)

GPIO.setup(25,GPIO.OUT)

# tell the GPIO library to look out for an

# event on pin 23 and deal with it by calling

# the buttonEventHandler function

GPIO.add_event_detect(23,GPIO.FALLING)

GPIO.add_event_callback(23,buttonEventHandler,100)

# turn off both LEDs

GPIO.output(25,False)

GPIO.output(24,True)

# make the red LED flash

while True:

GPIO.output(24,True)

time.sleep(1)

GPIO.output(24,False)

time.sleep(1)

GPIO.cleanup()

if __name__=="__main__":

main()

if __name__ == '__main__' est un un idiome qu’on voit souvent dans les scripts Python.

http://raspberrywebserver.com/cgiscripting/sending-data-to-an-HTTP-server-get-and-post-methods.html

http://sourceforge.net/p/raspberry-gpio-python/wiki/Inputs/

  sudo apt-get install alsa-utils

  $ sudo apt-get install mpg321

  sudo modprobe snd_bcm2835

  $ sudo amixer cset numid=3 1

GPIO.setup(23, GPIO.IN, pull_up_down=GPIO.PUD_DOWN)

GPIO.setup(24, GPIO.IN, pull_up_down=GPIO.PUD_UP)

# A rising-edge is defined by the time the pin changes from low to high, but it only detects the change.

# Similarly, the falling-edge is the moment the pin changes from high to low.

GPIO.wait_for_edge(23, GPIO.RISING)

GPIO.wait_for_edge(24, GPIO.FALLING)

GPIO.add_event_detect(23, GPIO.RISING, callback=printFunction, bouncetime=300)

?????? GPIO.add_event_callback

GPIO.cleanup()

Events let you remove them from a pin just as easily as you can add them :

GPIO.remove_event_detect(23)