http://rtldigitaldesign.blogspot.fr/2014/06/pos-n-neg-edge-detector.html

http://www.fpga4student.com/2017/02/verilog-code-for-d-flip-flop.html

https://www.boldport.com/blog/2015/4/3/edge-detect-ad-nauseam   super

// edge detect module

module edge_detect(

input  CLK,

input  RST,

input  IN,

output OUT

);

reg a, b;

// the edge detect signal is (b AND (NOT a))

assign OUT = a & !b;

always @(posedge CLK) begin

// It's always good to have a reset condition, otherwise

// the state of the register will show up as undertemined

// in simulation ('x')

if (RST == 1'b1) begin

a <= 0;

b <= 0;

end

else begin

a <= IN;

b <= a;

end

end

endmodule

///// http://www.edaboard.com/thread14551.html

https://github.com/pConst/basic_verilog/blob/master/EdgeDetect.v