Designing a synchronous finite state machine (FSM) is a common task for a digital logic engineer.

A finite state machine is a hardware component that advances from the

current state to the next state at the clock edge. This section discusses

methods and strategies for state machine encoding.

State Encoding Methods for State

Machines

There are several ways to encode a state machine, including sequential

(binary), gray code, and one-hot encoding. State machines with sequentially

encoded states have minimal numbers of flip-flops and wide combinatorial

functions. However, most FPGAs have many flip-flops and relatively narrow

combinatorial function generators. Sequential or gray-code encoding

schemes can result in inefficient implementation in terms of speed and

density for FPGAs. On the other hand, a one-hot encoded state machine

represents each state with one flip-flop. As a result, it decreases the width of

combinatorial logic, which matches well with register rich FPGA architectures.

In general, for FPGA Architectures, sequential or gray code encoding results

in a smaller area implementation and one-hot encoding results in a faster

implementation. For small state machines, less than 5 states, sequential is

typically the default because it only requires two bits so decoding the state is

fairly minimal. For larger state machines, 5 states or greater, one-hot is the

default because even though each state requires a register, the states do not

need to be decoded allowing for better performance.

There are various reasons you may choose to invoke a particular state

machine encoding scheme for a design. For example, you may choose to use

sequential encoding on a large state machine because performance in not

critical and you need to use a lot of register elsewhere in your design. You can
hard code the states in the source code by specifying a numerical value for

each state. This approach ensures the correct encoding of the state machine

but is more restrictive in the coding style. Alternatively, the enumerated coding

style leaves the flexibility of state machine encoding to the synthesis tools.

Most synthesis tools allow you to define encoding styles either through

attributes in the source code or through the tool’s user interface. Each

synthesis tool has its own synthesis attributes and syntax for choosing the

encoding styles. Refer to your synthesis tool’s documentation for details about

attributes syntax and values.

General State Machine Description

Generally, there are two approaches to describing a state machine.

One approach is to use one process or block to handle both state transitions and state outputs.

The other is to separate the state transition and the state outputs into two different processes or blocks.

The latter approach is more straightforward, because it separates the synchronous state registers from

the decoding logic that is used in the computation of the next state and the outputs.

https://pdfs.semanticscholar.org/fb6e/f3e30d912031f76ce1725e24546149ee9339.pdf

https://github.com/NetFPGA/netfpga/wiki/VerilogCodingGuidelines

Basically a FSM consists of combinational, sequential and output logic. Combinational logic is used to decide the next state of the FSM, sequential logic is used to store the current state of the FSM. The output logic is a mix of both combo and seq logic.

Types of State Machines

There are many ways to code these state machines, but before we get into the coding styles, let's first understand the basics a bit. There are two types of state machines:

Mealy State Machine : Its output depends on current state and current inputs

Moore State Machine : Its output depends on current state only.

Depending on the need, we can choose the type of state machine. In general, or you can say most of the time, we end up using Mealy FSM.

Encoding Style

Since we need to represent the state machine in a digital circuit, we need to represent each state in one of the following ways:

Binary encoding : each state is represented in binary code (i.e. 000, 001, 010....)

Gray encoding : each state is represented in gray code (i.e. 000, 001, 011,...)

One Hot : only one bit is high and the rest are low (i.e. 0001, 0010, 0100, 1000)

One Cold : only one bit is low, the rest are high (i.e. 1110,1101,1011,0111)

http://www.asic-world.com/tidbits/verilog_fsm.html

https://pdfs.semanticscholar.org/fc86/c86da97e1c56eb791728b5d072ae6b6d1c4f.pdf

Guideline: make state assignments using parameters with symbolic state names.

parameter [4:0] IDLE, S1, S2 , S3, ERROR;

IDLE = 5'b00001,

S1 = 5'b00010,

S2 = 5'b00100,

S3 = 5'b01000,

ERROR = 5'b10000;

Two-Always Block State Machine

A synthesizable state machine may be coded many ways.

Two of the most common, easily understood and efficient methods are two-always block and one-always block state machines.

The easiest method to understand and implement is the two-always block state machine with output assignments included in either the combinational next-state always block or separate continuous-assignment outputs. This method partitions the Verilog code into the major FSM

blocks shown in Figure 1: clocked present state logic, next state combinational logic and output

combinational logic.

We need register to hold the current state

We need next state function

https://courses.cs.washington.edu/courses/cse370/10sp/pdfs/lectures/15-VerilogIIPrint.pdf

- 4-State Mealy State Machine

The outputs of a Mealy state machine depend on both the inputs and the current state. When the inputs change, the outputs are updated without waiting for a clock edge.

- 4-State Moore State Machine

The outputs of a Moore state machine depend only on the present state. The outputs are written only when the state changes (on the clock edge).

http://www.sunburst-design.com/papers/CummingsICU2002_FSMFundamentals.pdf

always@ blocks are used to describe events that should happen under certain conditions.

always@ blocks are always followed by a set of parentheses, a

begin , some code, and an end

always@( ... sensitivity list ... )

begin

... elements ...

end

always@(posedge Clock)Blocks

Are used to describe sequential Logic, or registers.

Only <= (non-blocking) assignments should be used

Never use = (blocking) assignments

Only use these blocks when you want to infer an element(s) that changes its value at the positive or negative edge of the clock.

always@( * )Blocks

are used to describe Combinational Logic

, or Logic Gates.

Only = (blocking) assignments should be used . Never use <=(non-blocking) assignments

Only use when you want to infer an element(s) that changes its value as soon as one or more of its inputs change.

use ‘*’ (star) for your sensitivity list in block

The sensitivity list specifies which signals should trigger the elements inside the block to be updated.

If an sensisitit list signa change the value of every element insode the block are re evaluated

///////////////////////////////////////////////////////

https://github.com/NetFPGA/netfpga/wiki/VerilogCodingGuidelines

Verilog Coding guidelines

Signal naming

Explicit name
All signals are lowercase alpha, numeric and underscore only
Buses are numbered high order to low order: bus[31:0]
Array locations are numbered low to high. e.g. 2048x32 bit RAM:

reg [31:0] mem_array [0:2047];

Do use the same name for the signal at all levels of hierarchy

### Constant naming

Use capitals to identify constants: e.g. DEBUG , IDLE

## Indenting

Use three spaces per indentation level. Expand tabs to spaces to prevent indentation changing based upon editor settings.

if (this) begin

do something;

end

else begin

do something else;

end

## Commenting

- Comment your code. Someone will be coming back to this code in 6 months time, and that person will probably be you .

- When entering multi-line comments, try to use the "/*..*/" commenting rather than putting "//" in front of every line. It makes editing much easier.

- Add the following header to the top of all source code written by us (change the details as necessary):

- ///////////////////////////////////////////////////////////////////////////////

- // vim:set shiftwidth=3 softtabstop=3 expandtab:

- // $Id: reg_file.v 916 2006-04-20 23:23:25Z bob $

- //

- // Module: reg_file.v

- // Project: CPCI (PCI Control FPGA)

- // Description: Register file for access via PCI

- //

- //

- // Change history: 8/18/04 - Implemented DWORD0 (Revision/Version ID)

- //                 10/29/04 - Made the RESET signal last longer than

- //                 a single clock

- //                 01/08/05 - dma_rd_mac is now an input as round-robin

- //                 lookup is used

- //                 01/13/05 - split dma_intr into read and write sigs

- //

- ///////////////////////////////////////////////////////////////////////////////

## Module declaration/instantiation

Each module should be defined in a separate fime

Use Verilog 2001 style port declarations

Declare inputs and outputs one per line

Group signals logically by their function

Don't instantiate modules using positional arguments. Always use the dot form:

my_module my_module(

.signal (signal),

.a_bus  (a_bus),

...

);

## Assignment statments

- Sequential elements MUST only have non-blocking assignments (<=)

- Combinatorial should only have blocking assignments (=)

- Don't mix blocking and non-blocking assignments within a code block.

http://academic.csuohio.edu/chu_p/rtl/chu_rtL_book/rtl_chap10_fsm.pdf

http://tuline.com/wp-content/uploads/2015/11/Finite-State-Machine-Examples.pdf

http://www.fpga4student.com/2016/11/verilog-code-for-alarm-clock-on-fpga.html ++++++