http://madebyevan.com/fsm/

http://www.fizzim.com/examples/

http://www.graphviz.org/content/fsm

http://qfsm.sourceforge.net/download.html

http://fsme.sourceforge.net/

https://pdfs.semanticscholar.org/fc86/c86da97e1c56eb791728b5d072ae6b6d1c4f.pdf

https://inst.eecs.berkeley.edu/~cs150/sp12/resources/FSM.pdf

https://courses.cs.washington.edu/courses/cse370/10sp/pdfs/lectures/15-VerilogIIPrint.pdf

http://www.sunburst-design.com/papers/CummingsICU2002_FSMFundamentals.pdf

https://books.google.fr/books?id=DxQGrz7q-SwC&pg=PA56&lpg=PA56&dq=verilog+state+machine+with+loops&source=bl&ots=efhaWYuMAf&sig=lAurMiysqtO9H5y77Q0JF3OoJEE&hl=fr&sa=X&ved=0ahUKEwjbwv_C7c3XAhWELcAKHXVPBC0Q6AEIZjAH#v=onepage&q=verilog%20state%20machine%20with%20loops&f=false

https://github.com/stffrdhrn/adc_interface/blob/master/rtl/adc_interface.v

https://subodhtripathi.files.wordpress.com/2013/11/adc_interfacing.pdf

http://www.ti.com/lit/an/slaa199/slaa199.pdf

http://www2.dc.ufscar.br/~marcondes/netfpga/FPGAPrototypingByVerilogExamples.pdf

always @(posedge clk) begin

n <= n-2'h01;       -- thats the counter

if(n==2'h02) begin

m <=8'h02;

end

if(n==2'h01) begin

m <=8'h03;

end

if(n==2'h00) begin

m <=8'h04;

end

end

// 4-State Moore state machine

// A Moore machine's outputs are dependent only on the current state.

// The output is written only when the state changes.  (State

// transitions are synchronous.)

module moore_mac

(

input	clk, data_in, reset,

output reg [1:0] data_out

);

// Declare state register

reg		[1:0]state;

// Declare states

parameter S0 = 0, S1 = 1, S2 = 2, S3 = 3;

// Output depends only on the state

always @ (state) begin

case (state)

S0:

data_out = 2'b01;

S1:

data_out = 2'b10;

S2:

data_out = 2'b11;

S3:

data_out = 2'b00;

default:

data_out = 2'b00;

endcase

end

// Determine the next state

always @ (posedge clk or posedge reset) begin

if (reset)

state <= S0;

else

case (state)

S0:

state <= S1;

S1:

if (data_in)

state <= S2;

else

state <= S1;

S2:

if (data_in)

state <= S3;

else

state <= S1;

S3:

if (data_in)

state <= S2;

else

state <= S3;

endcase

end

endmodule

module statem(clk, in, reset, out);

input clk, in, reset;

output [3:0] out;

reg [3:0] out;

reg [1:0] state;

parameter zero=0, one=1, two=2, three=3;

always @(state)

begin

case (state)

zero:

out = 4'b0000;

one:

out = 4'b0001;

two:

out = 4'b0010;

three:

out = 4'b0100;

default:

out = 4'b0000;

endcase

end

always @(posedge clk or posedge reset)

begin

if (reset)

state = zero;

else

case (state)

zero:

state = one;

one:

if (in)

state = zero;

else

state = two;

two:

state = three;

three:

state = zero;

endcase

end

endmodule

assign sel_wire = {sel_3, sel_2, sel}; // concatenate

inputs for convenience

https://embeddedthoughts.com/2016/07/05/fpga-keyboard-interface/

http://www.referencedesigner.com/tutorials/verilogexamples/verilog_ex_10.php

sequencer

controller

https://www.cl.cam.ac.uk/teaching/2002/ECAD/ExtraExamples/ADCcontroller.pdf

http://referencedesigner.com/blog/level-change-detection-in-verilog/2663/

http://fpgacenter.com/examples/basic/edge_detector.php

always @(enable or lights_in) begin

#10

if (!( ( enable && lights_out = = lights_in) ||(!enable && lights_out = = 3'b000) ))

$display("Error in light controller output");

end

output that changes on the rising edge of CLK, ...

RUN1    6 conversion  et   enchainment  5  6    adc_pd remonte trop tot

Les autres RUNS  5 conversion     , la premiére 5 6 5 6  puis 5 6		adc_pd remonte trop tot

Premier enable  clock suivante ON

Les autres enable clock suivante ON puis enable passage 0 avant de revenir à 1