Afficheur LCD :2 lignes par 20 caractères type HD44780 
La puce de gestion du HD44780 possède 80 octets de RAM

Adressage ligne 1: 0,1,2,...19

Adressage ligne 1: 40,41,...59

Connexion du LCD:

Gnd(0V)= 1 

Vdd(5V)= 2

Potentiomètre pour ajuster le contraste (10K) 0 à +5V = 3

PA3 = 6 E (enable)

PA5 = 5 R/W (1 pour lire , 0 pour écrire)

PA4 = 4 RS (1 pour les datas, 0 pour le contrôle/état)

PD3-0 = 11-14 DB4-7 (4 bit data)

4-bit protocol read Busy

1) data direction input

2) R/W=1, RS=0

3) E=1

4) Wait a little time

5) Read 4-bit MS nibble data (bit 3 is busy)

6) E=0

7) E=1

8) Read 4-bit LS nibble data (nothing interesting)

9) E=0

10) R/W=0

11) direction=0x0F (default settings)
4-bit protocol write command (outCsr in LCD starter file)

0) wait for BUSY=0 (idle), fail if not idle within 2ms

1) set 4-bit MS nibble data port with command info

2) E=1

3) E=0

4) set 4-bit LS nibble data port with command info

5) E=1

6) E=0 (default settings)
4-bit protocol write ASCII data (LCD_OutChar in LCD starter file)

0) wait for BUSY=0 (idle), fail if not idle within 100us

1) RS=1

2) set 4-bit MS nibble data port with ASCII character

3) E=1

4) E=0

5) set 4-bit MS nibble data port with ASCII character

6) E=1

7) E=0

8) RS=0 (default settings)
