N-Channel versus P-Channel Input Stage

• Complimentary opamp can be designed with an n-channel input

differential pair and p-channel second-stage

• Overall gain would be roughly the same in both designs

P-channel Advantages

• Higher slew-rate: for fixed bias current, SR is larger (assuming

similar widths used for maximum gain)

• Higher frequency of operation: higher transconductance of

second stage which results in higher unity-gain frequency

• Lower 1/f noise: holes less likely to be trapped; p-channel

transistors have lower 1/f noise

• N-channel source follower is preferable (less voltage drop and

higher gm)

N-channel Advantage

• Lower thermal noise — thermal noise is lowered by high

transconductance of first stage