Nmos :

K = U0 . Cox		Tox = 3n

50-150 uA/V² is typical value of K for Tox ~ 100 angstrom. In your case Tox = 30 angstrom, thus you've obtained ~3 times larger value of K.

Cox=εox/tox

from the model file, for nominal process corner, the specified values are as follows
parameter NMOS PMOS

## 0.5 um AMI process

http://www.mosis.com/Technical/Testdata/ami_c5n_corner_bsim3.txt

AMI 0.5 um model Kp kn

tox 1.39e-8m 1.39e-8m
Cox(from tox) 2.484e-3F/m^2 2.484e-3F/m^2
un 458.43e-4m^2/Vs 212.02e-4m^2/Vs
Kn/Kp(=uncox) 113.88uA/V^2 52.66uA/V^2

http://fr.edaboard.com/topic-4305434.0.html

http://www.edaboard.com/thread142080.html

http://www.edaboard.com/thread44886.html

http://www.edaboard.com/thread147055.html

http://www.edaboard.com/thread57130.html

I have worked on TSMC 0.18u process following are the values u required :

Kn is between 182u to 225u (ss to ff)
Kp is between 54u to 65u (ss to ff)

Vthn = 0.68V to 0.88V (ff to ss)
Vthp = -0.6V to -0.8V (ff to ss)

all the parameters for 3.3V device.

Uo of NMOS is about 0.0459 m^2/Vs
Uo of PMOS is about 0.0109 m^2/Vs
vthn is about 0.445 (TT)
vthp is about -0.438 (TT)
for 1.8v MOSs

## process transconductance values Kp and Kn

NMOS PMOS

AMIS

1500n
K' (Uo*Cox/2) 36.6 -11.7 uA/V^2
500n
K' (Uo*Cox/2) 56.2 -18.9 uA/V^2
350n
K' (Uo*Cox/2) 93.1 -22.2 uA/V^2

TSMC

350n
K' (Uo*Cox/2) 92.9 -32.1 uA/V^2
250n
K' (Uo*Cox/2) 125.9 -25.9 uA/V^2
180n
K' (Uo*Cox/2) 171.9 -36.5 uA/V^2

IBM

500n
K' (Uo*Cox/2) 77.0 -20.9 uA/V^2
350n
K' (Uo*Cox/2) 91.4 -22.7 uA/V^2
250n
K' (Uo*Cox/2) 110.5 -27.0 uA/V^2
180n
K' (Uo*Cox/2) 153.7 -32.8 uA/V^2
130n
K' (Uo*Cox/2) 276.0 -47.6 uA/V^2

Transdiode

MOSFET fonctionnant en interrupteur :

NMOS current sink

On note VMIN la plage de tension qui correspond à un courant non constant. Le courant est dépendant de la tension ce qui n’est plus le cas d’un générateur de courant.

Si  et que l’on veut ID = 100µA  alors Rout = 250Kohm

NMOS cascoded current sink

Cette structure permet d’améliorer considérablement la résistance Rout