Eléments de technologies

VTH			 tension de seuil du NMOS 		[V]

VOV = VGS –VTH	 tension d’overdrive 			[V]

Kn = μn.Cox		 facteur de gain du NMOS 		[A/V2]

Uo			 mobilité des électrons			[cm2/Vs]

Cox			 capacité surfacique de grille 		[F/m2]

W 			 largeur de grille du transistor		[μm]

L 			 longueur de grille du transistor 		[μm]

λ = 1/VA		 modulation de la longueur du canal 	[V-1]

VA			 tension d’Early				[V]

1 Attention, il s’agit là d’un abus de langage, le phénom

Sachant que :

Tox (thickness of oxide)

Cox :

ou encore ou bien

pour le silicium (Sio2)

GPDK180  nmos

Uo de NMOS est d'environ 0,04 m²/Vs
Uo de PMOS est d'environ 0,01 m²/Vs

Je fixe W = L = 1µm , VGS = 1V et VDS = 3V

Sachant que:

Pour le NMOS :

ID = 41.469µA		VTH = 0.4795V		gds = 825.1E-9

Pour le PMOS :

ID = 7.924µA		VTH = 0.4563V		gds = 397.2E-9

////////////////////////////////////

Sachant que

NA : trous/m3
ND : electrons/m3
ni : intrinsèque carrier concentrationr	1.45E10 (cm3)

ND >> NA

/////////////////////////////////////////////////////

## channel length modulation

λ=sqrt(2ε/qNa) / (2*L*sqrt(Vds-Veff+phi))

ε = permitivity of the substrate
q = electron charge
Na = impurity concentration of the substrate
L = length
Veff = Vgs-Vt
phi = built in potential between diffusion and substrate

Simulating a single transistor will give you better results though.

## calculate channel length modulation

plot Id vs Vds for different Vgs and measure the slope of the curve in sat region and use lambda =(Id2-Id1)/(Vds2-Vds1)

lambda = (1/Id) * d(Id)/d(Vds).

TSMC018

Kn se situe entre 182u à 225u (SS à ff)
Kp est entre 54.U. à 65U (SS à ff)

Vthn = 0.68V à 0.88V (ff à ss)
Vthp =-0.6V to-0.8V (ff à ss)

Uo de NMOS est d'environ 0,0459 m ^ 2/Vs
Uo de PMOS est d'environ 0,0109 m ^ 2/Vs
vthn est d'environ 0.445 (TT)
vthp est d'environ -0,438 (TT)
1.8v pour les MOS

Kn = 405.36 cm^2/Vs * 0.4211 µAs/V(cm)^2 = 170.7 uA/V^2
Kp = 85.73 cm^2/Vs * 0.4211 µAs/V(cm)^2 = 36.1 uA/V^2

///// bizarre

Lambda = 1 /(rds.Ids)

1/(72.46E-3*41.469E-6)

////

lamda=gds/id

λ = 1 / (2*Rout*I)
Rout = gain / gm

Because Vgs=1v and (W/L)=1, we can get Id=17µA

17µA=K'n(1-0.536027)^2*(1+0.01*3) , so we can get K'n=76.67µA/V

ID = 41.469µA

K'n=(0.5)*U0*Cox=0.5*0.04660162 *3.46e-11/7.5e-9=107.49µA/V